Stopping Distance vs. Speed Lab

Names of people in your group:  __________________________________________

__________________________________________

__________________________________________

Consider the table below when you are asked to guess stopping distances.

Safe Following Distance

| Three-Second Rule |  | Safe Interval | Safe Interval |
| --- | --- | --- | --- |
|  |  | 3 seconds | 6 seconds |
| Speed |  | Good Weather | Marginal Weather |
| 25 m.p.h. | 37 ft. per second | 111 ft. | 222 ft. |
| 35 m.p.h. | 52 ft. per second | 166 ft. | 312 ft. |
| 45 m.p.h. | 66 ft. per second | 198 ft. | 396 ft. |
| 55 m.p.h. | 81 ft. per second | 243ft. | 486 ft. |
| 65 m.p.h. | 96 ft. per second | 288 ft. | 576 ft. |
| 75 m.p.h. | 111 ft. per second | 333 ft. | 666 ft. |

□	Each group will be assigned a specific coefficient of kinetic friction from 0.10 to 0.80.  Then students, as a group, must first fill in the guess column of the stopping distance in meters of the given speed at meters per second in the data table.

| µkinetic = ___ |  | Stopping Distance (Guess) | Stopping Distance (Guess) |
| --- | --- | --- | --- |
| Speed (m/s) | Speed (mi/hr) | Meters (m) | Feet (ft) |
| 5 | 11.2 |  |  |
| 10 | 22.4 |  |  |
| 15 | 33.5 |  |  |
| 20 | 44.7 |  |  |
| 25 | 55.9 |  |  |
| 30 | 67.1 |  |  |
| 35 | 78.3 |  |  |
| 40 | 89.4 |  |  |
| 45 | 100.6 |  |  |
| 50 | 111.8 |  |  |

As we have learned in our previous lessons whenever a moving object comes to a rest there has been a change in kinetic energy to a different form of energy such as potential energy, or as heat energy (created by work done by a frictional force on the rotors of the brakes which transfers to the tires).  Although the physics of stopping a traveling automobile in a real world situation is much more involved than the kinetic friction between the tires and the road surface this exercise will give us a very good estimation of understanding of stopping distances of an automobile.

Assuming proper operation of the brakes on a vehicle are constant, the minimum stopping distance for a moving vehicle is determined by the effective coefficient of friction between the tires and the road surface, and the driver's reaction time in a panic-braking situation. The frictional force between the road surface and the tires must do enough work on the car to reduce its kinetic energy to zero. If the wheels of the car continue to turn while braking, then static friction is in operation, BUT if the wheels are locked and they slide over the road surface (hydroplaning), the braking force is a kinetic friction force only.

We will assume that the wheels are locked, thus only the kinetic friction between the tires and the road surface is employed in this situation.

WNC = ΔKE + ΔU

WNC = Wfr = Ffr⋅d = FfrdcosθFfr,d

θFfr,d= 180o, cosθFfr,d= cos180o = -1

−Ffrd=KEf−KEi+0 , since  ΔU=0.

But KEf is also 0 because there is no final velocity, thus…

−Ffrd=−KEi

You can cancel out the negative signs from both sides of the equation.

Ffrd=KEi

But…

Ffr=μkFN=μkmg

And…

KEi=12mvi2

Thus by substituting the Ffr=μkmg and KEi=12mvi2…

μkmgd=12mvi2

Notice how the mass of the vehicle cancels out.  This implies a stopping distance independent of vehicle mass, and in this case.

μkgd=12vi2

Now if we solve for d…

d=vi22μkg

For calculating the minimum stopping distance, a value of 0.8 would be a nominal acceptable value for the coefficient of kinetic friction between good tires and a good road surface. Almost, if not always, coefficients of kinetic friction are less than the static coefficient of friction, and are significantly less for wet, slick, sandy, dirty, very smooth, oily, or icy surfaces. For most of the newer, high performance tires with good treads, the coefficient of kinetic friction and of static friction on a dry road surface may approach 0.8 if the braking is not so prolonged.

□	Using the following website

http://hpwizard.com/tire-friction-coefficient.html

for a tire type of High-Performance (HP) to estimate the kinetic coefficient of friction.

| Tire Condition | Road Condition | Estimated μk |
| --- | --- | --- |
| New HP Tires | Dry | 0.9 |
| HP Tires with 20% Tread wear | Dry |  |
| HP Tires with 50% Tread wear | Dry |  |
| HP Tires with 80% Tread wear | Dry |  |
| New HP Tires | Wet |  |
| HP Tires with 20% Tread wear | Wet |  |
| HP Tires with 50% Tread wear | Wet |  |
| HP with 80% Tread wear | Wet |  |
| New HP Tires | Snow |  |
| HP Tires with 20% Tread wear | Snow |  |
| HP Tires with 50% Tread wear | Snow |  |
| HP Tires with 80% Tread wear | Snow |  |
| HP Tires with 90% Tread wear | Icy |  |

□	Can you see how some coefficient of static friction overlaps depending on the tread wear and the road condition?

□	Now using the derived equation from above each group must calculate the stopping distance for each given speeds and the given coefficient of static friction and fill in the data table for the actual stopping distance.  Also transfer your guessed values on the same data table below.

d=vi22μkg

| µkinetic = ___ | Stopping Distance |  |
| --- | --- | --- |
| Speed (m/s) | Actual (m) | Guess (m) |
| 5 | _____ | _____ |
| 10 | _____ | _____ |
| 15 | _____ | _____ |
| 20 | _____ | _____ |
| 25 | _____ | _____ |
| 30 | _____ | _____ |
| 35 | _____ | _____ |
| 40 | _____ | _____ |
| 45 | _____ | _____ |
| 50 | _____ | _____ |

□	Now let’s see how correct we were with our stopping distance guesses.

First we will make a scatter plot of Guessed (y-axis) vs. Actual (x-axis) on MS Excel or on the graph paper provided by your instructor.

For the Excel users you must remember to place the independent variable (Actual) in the first column and the dependent variable (Guessed) in the second column.

Let’s think for a moment!

- If all of our guessed values is completely correct with the actual values what would the graph look like?

- If all of our guessed values is completely correct with the actual values what would the slope value be?

□	Now add a linear trend line for your scatter-plot and display the equation of your graph and the R2 value on your chart.

What is your slope value?   _____________________

Is it possible for you to get a negative slope value?  Please explain…

What does it mean if you get a huge slope value or a very small slope value?  Please explain…

Is it possible for you to get a ZERO slope value?  Please explain…

What is your R2 value?  ________________

What do you think the R2 value represents?

Comparing your data for the given Speeds and the Actual Stopping Distances could you estimate what kind of graph these data points will produce?  Please explain…

□	Now, lets make a scatter plot for the Actual Stopping Distance versus the Speed.  Since the speed is the independent variable (horizontal axis) you should place it in the first column for the Excel users.

What kind of graph is this?  Linear?  Quadratic?  Log?

If you chose quadratic, you must select the trend line of polynomial of order 2.

Using logic, how would you be able to convince someone that the y-intercept for your graph should be zero?  Please explain…

Using your graph, what is the stopping distance for 20 m/s?  ______

Again, using your graph, what is the stopping distance for 40 m/s?  ______

How much bigger (or how many times bigger) is the stopping distance for the 40 m/s when compared to the 20 m/s?

By looking at the derived equation, d=vi22μkg could you explain this?

Looking at the value of your stopping distance from your graph at 30 m/s could you estimate what the stopping distance might be for 60 m/s?  You can get the stopping distance value by hovering your mouse over the data point at 30 m/s.

Estimated stopping distance at 60 m/s:  _____________meters.

□	Please extrapolate (forecast forward) your graph 15 m/s on your trend line at the options menu.  Simply right click on your trend line and select [Format Trendline…].  Then click [Options] tab.  Find the [forecast] option and set the value to 15.  While you are here you can also set the y-intercept value to zero by just checking the [set the y-intercept to zero] box.

By looking at your new graph could you estimate what the stopping distance value is at 60 m/s?   ___________meters.

Is it about the same as your previously estimated value?  ________

What is the stopping distance for 20 m/s?  ______

What is the stopping distance for 60 m/s?  ______

How much bigger (or how many times bigger) is the stopping distance for the 60 m/s when compared to the 20 m/s?

By looking at the derived equation, d=vi22μsg could you explain this?

So…  What did we learn today by completing this exercise?

When you double your speed the stopping distance increases by ______ times the stopping distance of the original speed.

If you triple your speed then the stopping distance increases by ______ times the stopping distance of the original speed.

Therefore…

DRIVE SAFELY,

SLOW DOWN!!!

DON’T TAILGATE!!!

Especially with bad tires in a bad weather condition!!!

This knowledge may save your life or someone else’s!!!